| WG FM |  | Doc. FM(18)078R2 (FM57(18)003R2 – SE45(18)031R2 – SE19(18)022R2) |
| --- | --- | --- |
| 91st Meeting |  |  |
| Pratteln (near Basel) / Switzerland, 14 - 18 May 2018 |  |  |
| Date issued: | 1 June 2018 |  |
| Source: | ECO |  |
| Subject: | WGFM QUESTIONNAIRE TO CEPT ADMINISTRATIONS ON CURRENT AND FUTURE TRENDS FOR THE FIXED SERVICES AND STATUS OF OTHER INCUMBENT USES IN EUROPE IN THE FREQUENCY BAND 5925-6425 MHz |  |
| N Group membership required to read? (Y/N) |  |  |
| Summary: |  |  |
| WGFM#90 in February 2018 adopted this WGFM Questionnaire to CEPT Administrations. 37 CEPT Administrations provided an answer by 17 April 2018. Un update with additional information from France was received and incorporated in this document. Further detail on the number of FS links and FSS ES was provided by the United Kingdom, as included in the second revision of this document |  |  |
| Proposal: |  |  |
| - WGFM is invited to consider the results of the questionnaire in its 91st meeting in May 2018. - FM57 is requested to review the responses received and make proposals to WGFM. - SE19 and SE45 are invited to consider the results of the questionnaire as appropriate. |  |  |
| Background: The questionnaire as sent out by ECO is in Annex 1. The responses received are in Annex 2R1. This includes the received 28 excel spreadsheets and 20 word-file answers. The difference is due to the possibility to also provide only an electronic answer in the electronic questionnaire tool. Responses for Q4-Q7, as detailed by CEPT administrations in the table provided with the questionnaire, are compiled in the Excel spreadsheet in Annex 3R1. |  |  |

- Introduction

The questionnaire was submitted to the 48 CEPT countries. Contributing countries are highlighted in red in the Table below.

Table 1: Responses received (in red)

| Albania Andorra Austria Azerbaijan Belarus Belgium Bosnia Herzegovina Bulgaria Croatia Cyprus Czech Republic Denmark Estonia Finland France Georgia Germany Greece Hungary Iceland Ireland Italy Latvia Liechtenstein | Lithuania Luxembourg Former Yugoslavian Republic of Macedonia (FYROM) Malta Monaco Moldova Montenegro Norway Poland Portugal Romania Russian Federation San Marino Serbia Slovak Republic Slovenia Spain Sweden Switzerland The Netherlands The United Kingdom Turkey Ukraine Vatican City |
| --- | --- |

37 CEPT administrations provided a response.

FM57 and SE45 are in the process of performing a study in response to the Mandate to CEPT/ECC and collecting valid information regarding fixed service and other incumbent uses for possible co-existence with WAS/RLAN in the frequency band 5 925-6 425 MHz.

This information should be seen as complementary to the latest update of ECC Report 173. In this context WG FM consider that it would be useful for both industry and administrations to have a detailed and updated situation of the fixed service, and other incumbent uses, in Europe. This is of particular importance and timely taking into account new technology developments and requirements for these uses so that a complete picture is available of current and future spectrum use and requirements of the fixed service and other incumbent uses.

Main tasks of the WAS/RLAN 5 925-6 425 MHz study

The tasks of the Mandate to CEPT/ECC are:

Task 1 – Assessment and study of compatibility and coexistence scenarios in the band 5 925-6 425 MHz

To study and assess compatibility and coexistence scenarios for WAS/RLANs in the 5 925-6 425 MHz band and identify relevant parameters and coexistence conditions to be implemented in the regulatory framework in order to enable coexistence between existing usages and WAS/RLAN systems without constraining incumbent uses in various Member States in and adjacent to the band 5 925-6 425 MHz including at the outer EU border.

For each compatibility/coexistence scenario, the risk of interference, the deployment assumptions of all applications, the geographical extent of usage and consequential restrictions in WAS/RLAN deployment should be identified as well as requirements for implementing such scenarios, e.g. in terms of harmonised technical parameters or in terms of other regulatory and operational aspects which support the implementation of a coexistence framework.

Task 2 – Development of harmonised technical conditions

Taking into account the results of task 1, for the band 5 925-6 425 MHz develop appropriate mitigation techniques and/or operational compatibility/coexistence conditions. In the light of experience, these conditions should in particular identify the harmonised technical parameters that would be needed to ensure in the internal market consistent harmonised conditions for WAS/RLANs operating on a coexistence basis, if technically feasible. This should be developed in close cooperation with ETSI which is working on harmonised standards which include operational coexistence conditions for WAS/RLANs with other systems in the band and in adjacent bands.

It is assumed in the Mandate that WAS/RLANs could operate on the basis of a general authorisation only. With a view to achieving a scope for worldwide harmonisation of additional spectrum for WAS/RLAN that would strengthen the economies of scale for manufacturers of RLAN equipment and thereby benefit all end-users, the work carried out under this task should take into account developments in other ITU Regions, e.g. through the organisation of a workshop.

Purpose of the questionnaire

The questionnaire consisted of eight questions and the purpose is to collect information on the actual utilisation of the frequency band 5925-6425 MHz, where the fixed service is currently deployed and to gain an overview of the likely spectrum requirements in the future, as well as indication of the technology trends. This information will be used to analyse the current situation with respect to the development of the fixed service, and to allow a better vision of future deployment trends in order to study possible co-existence with WAS/RLAN.

It can be noted that fixed service characteristics and typical system parameters for sharing and co-existence studies can be found in Recommendation ITU-R F.758-6 and this questionnaire could be a measure to capture any other characteristics and parameters used within CEPT that are not covered by the range as stated in this recommendation.

Question 1: The total number of links in the frequency range 5925-6425 MHz?

|  | Respondent administration; Number of links. Albania; 139. Austria; 229. Azerbaijan; 74. Belarus; 0. Belgium; 226. Bosnia and Herzegovina; 46. Bulgaria; 231. Czech Republic; 159. Denmark; 10. Estonia; 0. Finland; 64. France; 1688. Georgia; 470. Germany; 784. Greece; 323. Hungary; 116. Iceland; 0. Ireland; 77. Italy; 1162. Latvia; 22. Lithuania; 13. Luxembourg; 0. FYROM; 41. Malta; 0. Montenegro; 132. Netherlands; 13. Norway; 811. Portugal; 20. Russian Federation; 2363. Slovakia; 70. Slovenia; 10. Spain; 850. Sweden; 466. Switzerland; 53. Turkey; 668. Ukraine; 57. United Kingdom; 351. TOTAL; 11738. |
| --- | --- |

Additional information under question 1:

|  | The total number of links in the frequency range 5925-6425 MHz? Please provide a numerical number |
| --- | --- |
| France | It should be noted that 14 links (understood as a bidirectional link using one frequency pair) are cross-Channel links between UK and France. |
| Denmark | Total number of fixed links is 10 including 6 offshore links (North Sea). |
| Georgia | About more than 470 duplex links have been already assigned to applicants. Main consumers are mobile operators. |
| Switzerland | The total number of 53 links is analysed. Increase of the number of links is envisioned in the next years. |
| United Kingdom | The UK provides details of its fixed links data under its “Open Data” policy. Where we can make the data available on an open basis, we use the Open Government Licence, which gives users various freedoms about how they choose to use the data, subject to some conditions. https://www.ofcom.org.uk/research-and-data/data/opendata#accordion__target-92004 The most recent export of spectrum detail can be found here; http://static.ofcom.org.uk/static/spectrum/wireless-telegraphy-register/WTR-Register-Report-12012018.xlsx Each licence number represents either a unidirectional or a bidirectional link. |

Question 2: Are the frequency usages distributed uniformly over the whole frequency range? and

Question 3: Are there any part(s) of the frequency band 5925-6425 MHz that are or may become more important than others?

Question 3 is complementary to Question 2. Consequently, almost no indication has been provided by administrations about certain sub-bands being more important than other sub-bands under question 3. The vast majority of answers from administrations is simply: ‘No’.

Uniform distribution?

| Distribution | Countries |
| --- | --- |
| Uniform distribution: Yes (23 countries) representing 9516 FS links | Albania, Austria, Azerbaijan, Belgium, Bosnia Herzegovina, Bulgaria, Czech Republic, France, Denmark, Georgia, Germany, Ireland, Italy, Macedonia (FYROM), Montenegro, Portugal, Russian Federation, Slovakia, Sweden, Switzerland, Turkey, Ukraine, United Kingdom |
| Distribution not uniform or special situation (11 countries representing 2222 FS links) Finland (64), Norway (811), Spain (850) Greece (323) Hungary (116) Latvia (22) Netherlands (13) Lithuania (13) Slovenia (10) | Distribution from the beginning of the band, i.e. first channels used first Not uniform but based on ERC/REC 14-01 all channels from 1 to 8 used All channels used but even channel numbers are about 3 times more used (ERC/REC 14-01) 5985-6425 MHz Channel 1 (29.65 MHz) not allocated, channels 2 to 5 assigned (ERC/REC 14-01) All links use centre frequency 5940 MHz 3 channels governmental use, remaining channels uniform distribution |
| No FS links (5 countries) | Belarus, Estonia, Iceland, Luxembourg, Malta (0) |
| Note that ERC/REC 14-01 includes a lower guard band in 5925-5930.375 MHz, a centre gap in 6167.575-6182.415 MHz and an upper guard band in 6419.615-6425 MHz For ECC/REC (14)06 (much less implementation than ERC/REC 14-01), the lower guard band is only 2.17 MHz wide and the centre gap is 6180.245-6424.553 MHz. From this summary, it can be concluded that the distribution over Europe is not uniform but it is also difficult to identify a clear trend indicating a major sub-band in 5925-6425 MHz with much lower use than within uniform distribution. |  |

The 29.65 MHz channelisation arrangement is nowadays the by far dominant use. Operators rather change the modulation than the bandwidth following their net capacity requirements. 59.3 MHz used for some high capacity links. Some additional use of narrower channels to be expected in the future.

Additional information under question 3:

|  | Please indicate which part(s) of the band, and reasons. See excel sheet in the introduction (Please indicate which part(s) of the band, possibly timeframes and reasons.) |
| --- | --- |
| France | The whole band is important. This band is used by fixed links over distances in the range of 30 to 60 km using the channeling arrangements defined in ERC Recommendation Rec 14-01. These links aggregate the traffic of tens of mobile base stations and are regularly upgraded to cope with the ever increasing traffic of the mobile networks. These links use a channel bandwidth of 29.5 MHz. Moreover, In order to implement the upcoming EC decision, harmonizing the L-band extensions for terrestrial systems capable of providing electronic communications services, France intends to migrate the narrowband fixed links currently operated in this band to the 5925 – 6425 MHz band following the channeling arrangements for the 6 GHz range based on ECC Recommendation 14(06) |
| United Kingdom | 5925-6425 MHz remains an important band for fixed links and our strategy is to continue enabling access of the whole band for fixed wireless links. Our strategy is also to explore smaller bandwidth channels (3.5 MHz and below) in the center gap and guard bands within the existing channel plans at 6 GHz for low capacity fixed links. |

Question 4: Please describe the distribution of links and their minimum, average, maximum link distance in densely, intermediate and sparsely populated areas respectively? Please indicate the distribution of usage given various geographical areas, ranging from city area to rural areas, in Table 1 (spreadsheet)

Information on the distribution of links in densely, intermediate and sparsely populated areas was provided in relation to 9586 FS links (out of a total number of 11738). Nearly half of FS links is located in sparsely populated areas, although the number of FS links in densely and intermediate populated areas is also significant. Note that the distribution varies country-by-country as mandated by the population distribution, size of the country and topology (e.g. Russian Federation is different compared with the Netherlands).

A detailed overview of the responses is provided below both in relation to the number and percentage of FS links.

The information on the minimum, maximum and average FS link distance in areas with dense, intermediate or sparse population density was provided by CEPT administration in relation to a set of 8440 links (out of a total of 11738).

The minimum and maximum FS link distances vary in the range indicated in the table below for areas with different population density:

| Population density | Range of minimum FS link distance (km) | Range of maximum FS link distance (km) |
| --- | --- | --- |
| Dense | 3 - 32 | 4.5 - 98.1 |
| Intermediate | 0.016 - 31.87 | 10 - 105.7 |
| Sparse | 5 - 60.5 | 30 - 122 |

The following figures represent the distribution of minimum and maximum FS link distances respectively, as provided by the respondent administrations:

The following figure represents the average FS link length (km) values in areas with different population density, as indicated by different CEPT administrations. Higher piles within a column indicate that corresponding average FS link distance indicated by an administration can be referred to a larger set of samples. The cardinality of each set of samples is reported below the figure.

|  | Considering the number of FS links and the corresponding average FS link distance indicated by the CEPT administrations, the weighted average values of FS link distance can be derived, obtaining: Weighted average FS link distance (km). Dense; 19.3. Intermediate; 24.0. Sparse; 34.2. |
| --- | --- |

Additional information has been provided by some administrations under question 4 on the distribution and length of FS links:

| Belgium | Minimum link distance = 8 km Maximum link distance = 108 km Average distance = 55 km |
| --- | --- |
| France | France provided additional statistics on the distribution and length of FS links: Type of area. Dense; Intermediate; Sparse; Very sparse. % of links; 3.96%; 9.17%; 46.88%; 39.98%. Minimum distance (km); 9; 8; 5; 2. Average distance (km); 29.37; 26.50; 26.38; 31.4. Median distance (km); 31; 24; 27; 30. Maximum distance (km); 49; 76; 92; 139. |
| Greece | EETT does not use population layers in the process of planning, studying and licensing of the PP links. The only relevant parameter kept in the link registry is whether the station is within a high or low population area, indicating the density of PP links (in total, not only for 5925-6425 MHz band), used for calculating spectrum fees. High population area is considered the prefecture of Attica (includes Athens) and all the other areas in Greece are considered as low populated. Therefore it is not feasible to provide a concrete numerical answer to this question. Nevertheless, please find below some similar statistics which may provide a clearer view to this direction: In the 3 largest cities of Greece there are only 29 links (9%) in this band with min / avg / max Distance 8,6 / 33,8 / 64 km. More specifically: - Athens: 6 links (only 2 are inside residential locations), min / max Distance 8,6 / 64 km - Thessaloniki 12 links (only 7 inside residential locations), min / max Distance 12 / 43 km - Patra: 11 links (only 3 are inside residential locations), min / max Distance 23,1 / 39,2 km - Rest of Greece: min / avg / max Distance 10,9 / 42,8 / 98,5 km The links not within residential locations include sites that are on top of near-city mountains or hills, in places surrounded by un-populated areas. The same applies also to links in other cities of Greece, therefore, links with sites inside dense or even intermediate population areas are very few. Moreover, a significant number of links are used for interconnecting islands. Taking into account that the majority of the cities of Greece ( >20.000 inhabitants) are located in areas with altitude less than 300m, the following statistics may provide useful information: - Avg site altitude for all 323 links: 668m - Links with at least one site with altitude >1000m: 67 (20,7%) - Links with at least one site with altitude >500m: 184 (57%) - Links with both sites altitude <500m: 75 (23,2%) - Links with both sites altitude <300m: 18 (5,6%) |
| Hungary | The distribution of link distance can be represented by normal distribution. The most common link distance is around 40 km. The number of links are lower when the link length is below or beyond this average value. Min. link distance: 16 km; Max. link distance: 65 km; Avg. link distance: 40 km. Most of the links are used for making long distance connection between two localities or a locality and a high terrain site. The links are deployed mostly in rural and sparsely populated areas and the link distance depends on the terrain. |
| Norway | Norway have not defined geographical areas and is not possible to Place. One cind of Applications is Communications between oil installations offshore. Important for long distance systems. |
| Portugal | The links are part of the backbone network, usually used to cover rural areas. |  |

Question 5: For Fixed Service, which Channel/Frequency plan is used (e.g. ERC/ECC Rec. ITU-R Rec.)? Please indicate used frequency plan and the distribution of channel bandwidths in Table 2 (spreadsheet).

The vast majority of CEPT administrations adopted ERC/REC 14-01 with channel bandwidth of 29.65 MHz (dominant) and 59.3 MHz, namely:

Albania, Austria, Belgium, Bosnia and Herzegovina, Bulgaria, Czech Republic, Denmark, Estonia, Finland, The Former Yugoslavian Republic of Macedonia (FYROM), France, Greece, Hungary, Ireland, Latvia, Luxembourg1, Malta1, Montenegro, The Netherlands, Norway, Portugal, Slovakia, Slovenia, Spain, Sweden, Switzerland, Turkey, Ukraine and United Kingdom.

Several CEPT administrations indicated either additional or alternative implementation of the FS links according to ITU-R Recommendation F.383-9, with reference to different channel bandwidth, namely:

5 MHz: Russian Federation

10 MHz: Austria

20 MHz: Azerbaijan and Lithuania

28 MHz: Austria, Georgia, Italy, Russian Federation and Ukraine

29.65 MHz: Germany, Italy, Montenegro, Russian Federation and Ukraine

59.3 MHz: Germany and Russian Federation

Austria, Slovenia, Malta and France have reported that implementation according to ECC/REC(14)06 is also allowed although no FS link has been deployed. Austria and France indicated that the arrangement provided by ECC/REC(14)06 is foreseen for a migration of narrow band links from the 1500 MHz range to the 6 GHz range.

| The information on the specific adopted channel arrangement and bandwidth was provided by CEPT administration in relation to a set of 8094 links (out of a total of 11738). More than a half of the FS links (5924) are deployed according to ERC/REC 14-01 with channels of 29.65 MHz; channel bandwidth of 59.3 MHz is also reported. As for the implementation of ITU-R Recommendation F.383-9, the most common bandwidth is again 29.65 MHz, but implementation with 20 MHz, 28 MHz and 59.3 MHz are also reported for a few hundreds of FS links. |  |
| --- | --- |

Additional information under question 5:

| United Kingdom | Please see further information and guidance contained in OfW48 (https://www.ofcom.org.uk/__data/assets/pdf_file/0016/92203/ofw48.pdf) UK Frequency Allocations for Fixed (Point-to-Point) Wireless Services and Scanning Telemetry and OfW446 (https://www.ofcom.org.uk/__data/assets/pdf_file/0017/92204/ofw446.pdf) Technical Frequency Assignment Criteria for Fixed Point-to-Point Radio Services with Digital Modulation. |
| --- | --- |

Question 6: What is the distribution of the technical parameters modulation rate, antenna gain, antenna height, transmit power at antenna port? Please indicate distribution in Tables 3A-3D (spreadsheet), please also indicate if adaptive modulation and/or automatic transmit power control is allowed and in use.

The information on the specific adopted modulation rate was provided by CEPT administration in relation to a set of 7355 FS links (out of a total of 11738). Almost one third of the FS links adopts a 128QAM modulation rate. Further details are shown in the following figures with reference to both the number and percentage of FS links.

Information on the implementation of adaptive modulation was provided only by a small number of CEPT administrations (see the table below), in relation to 335 FS links (out of a total of 11738).

Hungary, Spain, Sweden informed that adaptive modulation is allowed but not mandatory.

| Country | number of FS link with adaptive modulation | % of FS link in the country with adaptive modulation |
| --- | --- | --- |
| Albania | 25 | 18% |
| Greece | 119 | 36.8% |
| Montenegro | 132 | 100% |
| Portugal | 0 | 0 |
| Slovakia | 51 | 72.7% |
| Slovenia | 8 | 80% |
| Switzerland | 0 | 0 |
| TOTAL | 335 | -- |

Additional information under question 5, in relation to adaptive modulation:

| Slovenia | Allowed technics (adaptive modulation, automatic transmit power control, etc.) should be used as specified in ETSI EN 302 217. |
| --- | --- |
| Greece | Adaptive Modulation is allowed and it is currently licensed with reference Modulation of 128-QAM unless the network operator indicates a smaller number. Before this regulation, it was licensed with reference Modulation the max capable of the equipment again unless the network operator indicated a smaller number. Thus, inside the numbers provided in the respective table, Adaptive Modulation equipment is also included. More specifically 11/11 links with 16-QAM, 92/294 links with 128-QAM and 16/16 links with 256-QAM are using Adaptive Modulation (in total 119 links – 36,8%) |
| Czech Republic | Planning parameters of the links i.e. modulation rate, antenna gain, fade margin are exclusively within providers authority with respect to the frequency arrangement described in the part of Radio spectrum utilisation plan No. PV-P/19/12.2017-11, URL: https://www.ctu.eu/plan-use-radio-spectrum The Office registers antenna height and transmit power at antenna port only. |

The information on the FS link antenna gain was provided by CEPT administration in relation to a set of 10499 FS links (out of a total of 11738). Nearly 85% of the values for FS links antenna gain is within 33 and 45 dBi. Further details are shown in the following figures with reference to both the number and percentage of FS links.

The information on the FS link antenna height was provided by CEPT administration in relation to a set of 9136 FS links (out of a total of 11738). Details are shown in the following figures with reference to both the number and percentage of FS links.

The information on the FS link transmit power was provided by CEPT administration in relation to a set of 10530 FS links (out of a total of 11738). Almost half of the FS links have transmit power values between 29 and 32 dBm. Further details are shown in the following figures with reference both to the number and percentage of FS links.

Additional information under question 5, in relation to transmit power:

| France | Additional information was provided on: Minimum output power (dBm); 13. Average output power (dBm); 29. Median output power (dBm); 30. Maximum output power (dBm); 34. Minimum EIRP (dBm); 46. Average EIRP (dBm); 61.5. Median EIRP (dBm); 62. Maximum EIRP (dBm); 75. |
| --- | --- |

Information on the implementation of Automatic Transmit Power Control (ATPC) was provided only by a small number of CEPT administrations (see the table below) in relation to 1033 FS links (out of a total of 11738).

In addition Hungary, Spain, Sweden informed that ATPC is allowed but not mandatory; Czech Republic and Greece reported that ATPC is allowed but the no specific information is registered in the license.

Slovenia clarified that allowed technics (adaptive modulation, automatic transmit power control, etc.) should be used as specified in ETSI EN 302 217.

| Country | number of FS link with ATPC | % of FS link in the country with ATPC |
| --- | --- | --- |
| Albania | 17 | 12% |
| Bosnia and Herzegovina | 4 | 8.7% |
| Bulgaria | 116 | 50% |
| France | 219 | 13% |
| Germany | 421 | 53.7% |
| Montenegro | 132 | 100% |
| Portugal | 0 | 0 |
| Slovakia | 51 | 72.7% |
| Slovenia | 10 | 100% |
| Switzerland | 53 | 100% |
| TOTAL | 1023 |  |

Question 7: What is the minimum fade margin respectively maximum fade margin allowed? Please also indicate distribution of fade margin in Table 4 (spreadsheet).

The information on the minimum fade margin respectively maximum fade margin allowed was provided by CEPT administration in relation to a set of 4047 FS links (out of a total of 11738). For two thirds of the FS links this information was not available. Further details are shown in the following figures with reference to both the number and percentage of FS links.

In addition Spain and Greece informed that there are no restrictions in relation to the fade margin

Additional information under question 7:

| The Netherlands | Fade Margin is calculated, depending on the path length of the radio-link. |
| --- | --- |
| Switzerland | Note: Nominal RX input level = - 50 dBm, RX input level tolerance: +1 (for long links) to -10 dB (for short links) with min. ATPC TX output power under clear sky conditions.. 18 dBm. |

Question 8: What other usages should be considered in the frequency range 5925-6425 MHz? Please indicate which application(s) (e.g. Communications-based train control, satellite earth station, etc.), in which parts of the band, the geographical areas, the number of sites and other general radio characteristics.

FSS

Some numbers for FSS earth stations have been provided: Austria (6), Finland (‘only few’), France (120), Germany (15), Hungary (2), Latvia (2), Lithuania (1), Luxembourg (3 major sites), Portugal (8), Slovenia (2), Spain (9), Sweden (15), Switzerland (1) and United Kingdom (approximatively 90). In addition, there is considerable use by a higher number of FSS stations in the Russian Federation (number unknown).

Some of these FSS earth stations are concentration at a limited number of FSS hub-stations/gateways. Some FSS use may actually be military or governmental use on a national basis.

Information about the location of FSS earth stations has been provided, with different level of details, by Austria, Greece, Latvia, Luxembourg, Portugal, Russian Federation and Turkey.

There are also 7 ESV networks notified to the Office (ECC/DEC/(06)09).

There are passive sensors (satellite) used for sea surface temperature, sea surface wind speed and soil moisture measurements.

Urban Rail CBTC

Used in metropolitan cities in France (5915-5935 MHz) and Denmark (up to 5975 MHz).

France reported of several metro lines equipped in Paris and about many new projects ongoing.

License-exempt use as covered by UWB (EC Decision for UWB and Rec. 70-03) and SRD (EC DEC for SRD and Rec. 70-03) has been mentioned.

Some ENG/OB links used as reported by Greece. They may have been counted for some other countries as FS (question 8 is about ‘other use´). Italy has also ENG/OB in their frequency utilisation plan. In the Russian Federation, the band 5925-6425 MHz is heavily used by fixed wireless access systems (P-P, P-MP) (may also be counted by other administrations under FS).

- Information in EFIS

The information in EFIS for ECO Report 04 has been updated by the Office.

Information about fixed service implementation over the whole range 5925-6425 MHz is available for the following countries which have not provided a response to the questionnaire: Croatia, Cyprus, Moldova, Poland, Romania and Serbia. This means that the total number of FS links over Europe could be somewhat higher in 5925-6425 MHz.

- ECC Report 173

The data received under this questionnaire seems to be consistent with the information in ECC Report 173, especially with regard to the overall usage in Europe (approximate number of FS links), link hop lengths etc.

_________________

Estonia, Luxembourg and Malta provided information although no FS link is deployed.