CURRENT AND FUTURE TRENDSFOR THE FIXED SERVICES AND STATUS OF OTHER INCUMBENTUSES IN EUROPE IN THE FREQUENCY BAND 5925-6425 MHz

QUESTIONNAIRETO CEPT ADMINISTRATIONS

In order for your views to be taken into account in this study, please return replies to Thomas Weber, e-mail: thomas.weber@eco.cept.org until 12 April 2018

Electronic questionnaire Link

To: 			Mr Thomas Weber

preferably by e-mail:	thomas.weber@eco.cept.org

or

by fax:			+45 33 89 63 30

| Administration/name of organisation: | Ukraine/State Enterprise “Ukrainian State Centre of Radio Frequencies” |
| --- | --- |
| Name of responsible person: | Igor Gepko |
| Phone: | +380 44 422 85 67 |
| Fax: | +380 44 422 81 81 |
| E-mail: | gepko@ucrf.gov.ua |
| Date: | 20 March 2018 |

Question 1: The total number of links in the Frequency range 5925-6425 MHz?Please provide a numerical number (one link should be understood as a bidirectional link using one frequency pair).

57 links in total*

Question 2: Are the frequency usages distributed uniformly over the whole frequency range? Free text, please answer either YES or provide more information in case of any non-uniform distribution.

The frequency usages distributed nearly uniformly over the whole frequency range(with a minor trend for more intense use of its lower part)

Question 3: Are there any part(s) of the frequency band 5925-6425 MHz that are or may become more important than others? Please indicate which part(s) of the band, and reasons.

All parts are nearly the same in value

Question 4: Please describe the distribution of links and their minimum, average, maximum link distance in densely, intermediate and sparsely populated areas respectively? Please indicate the distribution of usage given various geographical areas, ranging from city area to rural areas, in Table 1

These links are used in 9 regions of Ukraine (approximately 1/3 of its territory). Less than 40% of themare located in densely populated (urban) areas and slightly more than 60% are located in intermediate populated areas.

Question 5: For Fixed Service, which Channel/Frequency plan is used (e.g. ERC/ECC Rec. ITU-R Rec.)? Please indicate used frequency plan and the distribution of channel bandwidths in Table 2. * ERC Recommendations 14-01 /14-06 or an alternative arrangement.

Please see Table 2.

Question 6: What is the distribution of the technical parameters modulation rate, antenna gain, antenna height, transmit power at antenna port? Please indicate distribution in Tables 3A-3D, please also indicate if Adaptive modulation and/or Automatic Transmit Power Control is allowed and in use.

Please see Tables 3A-3D.

Question 7: What is the minimum fade margin respectively maximum fade margin allowed? Please also indicate distribution of fade margin in Table 4.

Data not available.

Question 8: What other usages should be considered in the frequency range 5925-6425 MHz? Please indicate which application(s) (e.g. Communications-based train control, satellite earth station, etc.), in which parts of the band, the geographical areas, the number of sites and other general radio characteristics.

* This frequency range may also be used by special users of spectrum

ANNEX 1

| Table 1, Distribution of links |  |  |  |
| --- | --- | --- | --- |
|  |  | Type of Area |  |
| Parameter | Dense | Intermediate | Sparsely |
| Percentage of links (%) | 37 | 63 |  |
| Minimum Distance (km) | 25 | 19,2 |  |
| Average Distance (km) | 44,4 | 43,2 |  |
| Maximum Distance (km) | 62,4 | 52,5 |  |
| Table 2, Distribution of channels |  |  |  |
|  | Links using specific channel spacing |  |  |
| Channel bandwidth (MHz) | ERC/RECs 14-01/14-06 | Rec. ITU-R F.383-9 |  |
| 0,025 |  |  |  |
| 0,25 |  |  |  |
| 0,5 |  |  |  |
| 1,75 |  |  |  |
| 3,5 |  |  |  |
| 5 | N/A |  |  |
| 10 | N/A |  |  |
| 20 | N/A |  |  |
| 28 | N/A | 15 |  |
| 29.65 | 40 | 2 |  |
| 40 | N/A |  |  |
| 59.3 |  |  |  |
| Table 3A, Modulation rate |  |  |  |
| Modulation rate | Links (%) |  |  |
| analog FM | 0,7 |  |  |
| QPSK | 0,25 |  |  |
| 4QAM | 0 |  |  |
| 16QAM | 0 |  |  |
| 32QAM | 0 |  |  |
| 64QAM | 0 |  |  |
| 128QAM | 0,05 |  |  |
| 256QAM | 0 |  |  |
| 512QAM | 0 |  |  |
| 1024QAM | 0 |  |  |
| Adaptive Modulation | 0 |  |  |
| Table 3B, Antenna gain |  |  |  |
| Antenna gain range (dBi) | Links (%) |  |  |
| up to 30 | 0 |  |  |
| 30-33 | 0 |  |  |
| 33-36 | 0 |  |  |
| 36-39 | 8,8 |  |  |
| 39-42 | 24,5 |  |  |
| 42-45 | 66,7 |  |  |
| above 45 | 0 |  |  |
| Table 3C, Antenna Height |  |  |  |
| Antenna height range (m) | Links (%) |  |  |
| below 20m | 1,7 |  |  |
| 20-40 m | 5,3 |  |  |
| 40-60 m | 19,3 |  |  |
| 60-80 m | 39,5 |  |  |
| 80- 100 m | 21 |  |  |
| above 100 m | 13,2 |  |  |
| Table 3D, Transmit power at antenna port |  |  |  |
| Transmit power range (dBm) | Links (%) |  |  |
| below 20 dBm | 0 |  |  |
| 20-23 dBm | 2 |  |  |
| 23-26 dBm | 10,5 |  |  |
| 26-29 dBm | 14 |  |  |
| 29-32 dBm | 10,5 |  |  |
| above 32 dBm | 63 |  |  |
| ATPC |  |  |  |
| Table 4, Fade margin |  |  |  |
| Fade margin range (dB) | Links (%) |  |  |
| below 20 dB | N/D |  |  |
| 20-25 dB | N/D |  |  |
| 25-30 dB | N/D |  |  |
| 30-35 dB | N/D |  |  |
| 35-40 dB | N/D |  |  |
| 40-45 dB | N/D |  |  |
| above 45 dB | N/D |  |  |