FM(18)059 - Annex 23

FM(18)059 - Annex 23

| Administration/name of organisation: | MINISTRY OF DIGITAL POLICY, TELECOMMUNICATIONS & MEDIA GENERAL SECRETARIAT OF TELECOMMUNICATIONS & POSTS DIRECTORATE OF RADIOSPECTRUM MMANAGEMENT |
| --- | --- |
| Name of responsible person: | Spyridon Chatziloizos |
| Phone: | 0030 – 210 6508554 |
| Fax: | 0030 – 210 6508560 |
| E-mail: | s.chatz@yme.gov.gr |
| Date: | 13/04/2014 |

Questions for Administrations only on the current national practices for allocation and assignment of fixed service frequencies/licences and future trends/requirements:

Question 1: The total number of links in the Frequency range 5925-6425 MHz? Please provide a numerical number

323

Question 2: Are the frequency usage distributed uniformly over the whole frequency range? Free text, please answer either YES or provide more information in case of any non-uniform distribution

Taking into account the channel numbering described in ERC/REC 14-01 (Annex 1) for bandwidth 29,65 MHz (100% of the links), the distribution is as follows: Channel 1 (66 Links – 56 V / 10 H) Channel 2 (41 Links – 23 V / 18 H) Channel 3 (58 Links – 48 V / 10 H) Channel 4 (35 Links – 16 V / 19 H) Channel 5 (40 Links – 32 V / 8 H) Channel 6 (31 Links – 16 V / 15 H) Channel 7 (32 Links – 27 V / 5 H) Channel 8 (20 Links – 12 V / 8 H) It is obvious that the distribution is not uniform over the whole frequency range. The first half of the band hosts 200 Links in comparison with 123 of the second half. Moreover, the odd channels host 196 links in comparison with 127 links of the even channels. The reason for this distribution depends on some details in the procedure that the network operators follow in order to select the channel preferences (which is accepted by EETT if available). The specific steps are mainly the following (confirmed by greek network operators) - The capacity demand drives to the number of channels required - Lower frequencies (even 100-200 MHz lower) and vertical polarisation have a bit higher link performance - Depending on the demand, the operator starts to examine the channels with a specific order: 1V-3V-5V-7V. - If not available the next choice is 2V-4V-6V-8V - The horizontal polarisation is selected only if the availability is low or if the demand is too high and the vertical channels are not enough (e.g. 1V-2H-3V-4H-5V-6H-7V-8H) The above procedure fully explains the lack of uniformity in the frequency band parts used.

Question 3: Are there any part(s) of the frequency band 5925-6425 MHz that are or may become more important than others? Please indicate which part(s) of the band, and reasons.

Currently, there are no such parts of the band and there are no intentions in the near future to change this. All the band parts are equally important.

Question 4: Please describe the distribution of links and their minimum, average, maximum link distance in densely, intermediate and sparsely populated areas respectively? Please indicate the distribution of usage given various geographical areas, ranging from city area to rural areas, in table 1

EETT does not use population layers in the process of planning, studying and licensing of the PP links. The only relevant parameter kept in the link registry is whether the station is within a high or low population area, indicating the density of PP links (in total, not only for 5925-6425 MHz band), used for calculating spectrum fees. High population area is considered the prefecture of Attica (includes Athens) and all the other areas in Greece are considered as low populated. Therefore it is not feasible to provide a concrete numerical answer to this question. Nevertheless, please find below some similar statistics which may provide a clearer view to this direction: In the 3 largest cities of Greece there are only 29 links (9%) in this band with min / avg / max Distance 8,6 / 33,8 / 64 km. More specifically: - Athens: 6 links (only 2 are inside residential locations), min / max Distance 8,6 / 64 km - Thessaloniki 12 links (only 7 inside residential locations), min / max Distance 12 / 43 km - Patra: 11 links (only 3 are inside residential locations), min / max Distance 23,1 / 39,2 km - Rest of Greece: min / avg / max Distance 10,9 / 42,8 / 98,5 km The links not within residential locations include sites that are on top of near-city mountains or hills, in places surrounded by un-populated areas. The same applies also to links in other cities of Greece, therefore, links with sites inside dense or even intermediate population areas are very few. Moreover, a significant number of links are used for interconnecting islands. Taking into account that the majority of the cities of Greece ( >20.000 inhabitants) are located in areas with altitude less than 300m, the following statistics may provide useful information: - Avg site altitude for all 323 links: 668m - Links with at least one site with altitude >1000m: 67 (20,7%) - Links with at least one site with altitude >500m: 184 (57%) - Links with both sites altitude <500m: 75 (23,2%) - Links with both sites altitude <300m: 18 (5,6%)

Question 5: For Fixed Service, which Channel/Frequency plan is used (e.g. ERC/ECC Rec. ITU-R Rec.)? - Please indicate used frequency plan and the distribution of channel bandwidths in table 2.

As also expressed above, the channel plan used is ERC/REC 14-01 (Annex 1) for bandwidth 29,65 MHz. All 323 links follow this arrangement.

Question 6: What is the distribution of the technical parameters modulation rate, antenna gain, antenna height, transmit power at antenna port? Please indicate distribution in table 3A-3D, please also indicate if Adaptive modulation and/or Automatic Transmit Power Control is allowed and in use.

Clarifications regarding the numbers in the attachment: - Adaptive Modulation is allowed and it is currently licensed with reference Modulation of 128-QAM unless the network operator indicates a smaller number. Before this regulation, it was licensed with reference Modulation the max capable of the equipment again unless the network operator indicated a smaller number. Thus, inside the numbers provided in the respective table, Adaptive Modulation equipment is also included. More specifically 11/11 links with 16-QAM, 92/294 links with 128-QAM and 16/16 links with 256-QAM are using Adaptive Modulation (in total 119 links – 36,8%). - ATPC is allowed but not included in the license. The transmission power referred in the license is the maximum allowed for the specific link.

Question 7: What is the minimum fade margin respectively maximum fade margin allowed? Please also indicate distribution of fade margin in table 4.

There are no regulatory limitations regarding the min or max fade margin. The value of fade margin that can be extracted from the data stored in our technical database assumes a receive level that includes the effects of Rain Attenuation and Clear Air Fading. In this context, the distribution of values of fade margin are shown in the relative table in the accompanying spreadsheet.

Question 8: What other usages should be considered in the frequency range 5925-6425 MHz? Please indicate which application(s) (e.g. Communications-based train control, satellite Earth station, etc.), in which parts of the band, the geographical areas, the number of sites and other general radio characteristics

- There are licenses in this band for ENG/OB Vans in Athens (3) and Thessaloniki (1). The Vans could be located anywhere in the licensed area (fixed while transmitting) and the links have the following characteristics: - Unidirectional, Van (Tx) to Site (Rx) on a mountain - Analogue, 28 MHz bandwidth - Horizontal polarisation - Antennas of 0.6 m diameter, height 15-20 m and approx. 30 dBi Gain - Transmit power 30 dBm - There are Fixed Satellite transmissions (E-s) with the following characteristics: - 6 Earth Stations, 4 in Athens within residential areas and 2 in non-residential areas (satellite farms) - 17 different transmission frequencies within the band. 16/17 are in the part 6029-6350 MHz and only one at 5945 MHz - 16/17 transmissions have less than 1 MHz bandwidth and only one has 16 MHz - EIRP range: 50-75 dBW