CURRENT AND FUTURE TRENDS FOR THE FIXED SERVICES AND STATUS OF OTHER INCUMBENT USES IN EUROPE IN THE FREQUENCY BAND 5925-6425 MHz

QUESTIONNAIRE TO CEPT ADMINISTRATIONS

| Administration/name of organisation: | Agence nationale des fréquences (ANFR, France) |
| --- | --- |
| Name of responsible person: | Antone Borissov-Tete |
| Phone: | +33 / 1 45 18 73 45 |
| Fax: |  |
| E-mail: | antone.borisov-tete@anfr.fr |
| Date: | 16/04/2018, updated 25/04/2018 |

Questions for Administrations only on the current national practices for allocation and assignment of fixed service frequencies/licences and future trends/requirements:

Question 1: The total number of links in the Frequency range 5925-6425 MHz? Please provide a numerical number.

Total number of links: 1688 It should be noted that 14 of them are cross-Channel links between UK and France. Note: One link is understood as a bidirectional link using one frequency pair.

Question 2: Are the frequency usages distributed uniformly over the whole frequency range? Free text, please answer either YES or provide more information in case of any non-uniform distribution.

The frequency band is used uniformly.

Question 3: Are there any part(s) of the frequency band 5925-6425 MHz that are or may become more important than others? Please indicate which part(s) of the band, and reasons.

The whole band is of importance. This band is used by fixed links over distances in the range of 30 km, sometimes up to 100 km, using the channelling arrangement defined in ERC Recommendation 14-01. These links aggregate the traffic of tens of mobile base stations and are regularly upgraded to cope with the ever increasing traffic of the mobile networks. These links use a channel bandwidth of 29.5 MHz. Moreover, in order to implement the upcoming EC decision harmonizing the L-band for MFCN, France intends to migrate the narrowband fixed links currently operated in the L band to the 5925-6425 MHz band following the channelling arrangements for the 6 GHz range based on ECC Recommendation 14(06).

Question 4: Please describe the distribution of links and their minimum, average, maximum link distance in densely, intermediate and sparsely populated areas respectively? Please indicate the distribution of usage given various geographical areas, ranging from city area to rural areas, in Table 1.

Table 1

|  | Type of area |  |  |  |
| --- | --- | --- | --- | --- |
|  | Dense | Intermediate | Sparse | Very sparse |
| % of links | 3.96% | 9.17% | 46.88% | 39.98% |
| Minimum distance (km) | 9 | 8 | 5 | 2 |
| Average distance (km) | 29.37 | 26.50 | 26.38 | 31.4 |
| Median distance (km) | 31 | 24 | 27 | 30 |
| Maximum distance (km) | 49 | 76 | 92 | 139 |

Question 5: For Fixed Service, which Channel/Frequency plan is used (e.g. ERC/ECC Rec. ITU-R Rec.)? Please indicate used frequency plan and the distribution of channel bandwidths in Table 2.

Table 2

| Channel bandwidth (MHz) | % of links |
| --- | --- |
| 0.025 |  |
| 0.25 |  |
| 0.5 |  |
| 1.75 |  |
| 3.5 |  |
| 5 |  |
| 10 |  |
| 20 |  |
| 28 |  |
| 29.65 | 100% |
| 40 |  |
| 59.3 |  |

Question 6: What is the distribution of the technical parameters modulation rate, antenna gain, antenna height, transmit power at antenna port? Please indicate distribution in Tables 3A-3D, please also indicate if Adaptive modulation and/or Automatic Transmit Power Control is allowed and in use.

Table 3A

| Modulation rate | % of links |
| --- | --- |
| 4QAM | 0.06% |
| 16QAM | 0.18% |
| 32QAM | 0% |
| 64QAM | 0.65% |
| 128QAM | 33.77% |
| 256QAM | 9.48% |
| 512QAM | 49.76% |
| 1024QAM | 4.50% |
| Adaptive modulation | n/a |

Table 3B

| Antenna gain (dBi) | % of links |
| --- | --- |
| Gain ≤ 30 | 0% |
| 30 < Gain ≤ 33 | 1.72% |
| 33 < Gain ≤ 36 | 11.92% |
| 36 < Gain ≤ 39 | 52.39% |
| 39 < Gain ≤ 42 | 23.35% |
| 42 < Gain ≤ 45 | 10.57% |
| 45 < Gain | 0.06% |

Table 3D

| Output power (dBm) at antenna connector | % of links |
| --- | --- |
| P ≤ 20 | 1.03% |
| 20 < P ≤ 23 | 5.66% |
| 23 < P ≤ 26 | 13.38% |
| 26 < P ≤ 29 | 2.67% |
| 29 < P ≤ 32 | 79.61% |
| 32 < P | 4.18% |

| Minimum output power (dBm) | 13 |
| --- | --- |
| Average output power (dBm) | 29 |
| Median output power (dBm) | 30 |
| Maximum output power (dBm) | 34 |

| Minimum EIRP (dBm) | 46 |
| --- | --- |
| Average EIRP (dBm) | 61.5 |
| Median EIRP (dBm) | 62 |
| Maximum EIRP (dBm) | 75 |

Links using ATPC: 13%

Table 3C

| Antenna height (m) | % of links |
| --- | --- |
| H ≤ 20 | 26.84% |
| 20 < H ≤ 40 | 43.48% |
| 40 < H ≤ 60 | 21.03% |
| 60 < H ≤ 80 | 5.45% |
| 80 < H ≤ 100 | 1.24% |
| 100 < H | 2.01% |

Question 7: What is the minimum fade margin respectively maximum fade margin allowed? Please also indicate distribution of fade margin in Table 4.

Information not available.

Question 8: What other usages should be considered in the frequency range 5925-6425 MHz? Please indicate which application(s) (e.g. Communications-based train control, satellite earth station, etc.), in which parts of the band, the geographical areas, the number of sites and other general radio characteristics.

In France, this band is also used by FSS Earth stations (around 120). Some licences have been granted in the 5915-5935 MHz band for CBTC. Several metro lines are now equipped in Paris: line 3 since 2010, line 1 since 2011, line 5 since 2013 and line 9 since 2015. These lines are used by 2.1 million passengers and CBTC covers more than 62 km of rail tracks (without taking into account shunting yards). Many new projects are ongoing: - line B in Rennes for 2018 - line 4 in Paris for 2020 - line M1 in Lille for 2020 - line B in Lyon for 2020 - line 11 in Paris for 2022 - lines 1 and 2 in Marseille for 2022 - lines 6 and NextEo in Paris area for 2023 - line D in Lyon for 2024 - line 14 in Paris for 2024 For instance, the 5 projects for Paris area will result in 71 km of additional rail tracks covered by CBTC. By 2030, lines 10 and 12, all new lines 15, 16, 17 and 19 as well as the “Grand Paris Express” will also be covered by CBTC, which represent more than 200 km of rail tracks.

**end**